علیرضا سلطانی نشان | 98111033302016

درس سخت افزار

استاد افشاری

کد درس 2328

مثال اول) روشن کردن LED با استفاده از اعلان استفاده شده در پروتکل UDP، بطوری که turnon و turnoff اگر صدا شود چراغ توسط برد آردواینو و جواستیک و خازن روشن یا خاموش میشود.

#include <ESP8266WiFi.h>

#include <WiFiUdp.h>

const char* ssid = "sofa" ;const char* password = "AL20re/*";

WiFiUDP Udp;

unsigned int localUdpPort = 4210;  // local port to listen on

char incomingPacket[255];  // buffer for incoming packets

char  replyPacket[] = "Hi there! Got the message :-)";  // a reply string to send back

void setup()

{

Serial.begin(115200);

Serial.println();

Serial.printf("Connecting to %s ", ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

// initialize digital pin LED_BUILTIN as an output.

pinMode(LED_BUILTIN, OUTPUT);

Serial.println(" connected");

Udp.begin(localUdpPort);

Serial.printf("Now listening at IP %s, UDP port %d\n", WiFi.localIP().toString().c_str(), localUdpPort);

}

void loop()

{

int packetSize = Udp.parsePacket();

if (packetSize)

{

// receive incoming UDP packets

//    Serial.printf("Received %d bytes from %s, port %d\n", packetSize, Udp.remoteIP().toString().c_str(), Udp.remotePort());

int len = Udp.read(incomingPacket, 255);

if (len > 0)

{

incomingPacket[len] = 0;

}

Serial.println(incomingPacket[5]);

char turnOnChar [255] = "turnOn";

char turnOffChar [255] = "turnOff";

Serial.println(incomingPacket);

if (incomingPacket[5] == turnOnChar[5]) {

digitalWrite(LED_BUILTIN, LOW);

Serial.println("THE =>>> Turn On LED");

} else if (incomingPacket[5] == turnOffChar[5]) {

digitalWrite(LED_BUILTIN, HIGH);

Serial.println("THE =>>> Turn off LED");

}

//    Serial.printf("UDP packet contents: %s\n", incomingPacket);

// send back a reply, to the IP address and port we got the packet from

Udp.beginPacket(Udp.remoteIP(), Udp.remotePort());

Udp.write(replyPacket);

Udp.endPacket();

}

}

مثال دوم) ارسال اطلاعات جیسون به سمت برد که اطلاعات را به آن قالب توسط پروتکل HTTP دریافت میکند.

- /*

- *  ESP8266 JSON Decode of server response

- *  - Alireza Soltani Neshan | 98111033302016

- *  - project 5

- *  - IoT

- */

- #include <ESP8266WiFi.h>

- #include <WiFiClient.h>

- #include <ESP8266HTTPClient.h>

- #include <ArduinoJson.h>

- const char* wifiName = "iPhone";

- const char* wifiPass = "AL20re/*";

- //Web Server address to read/write from

- const char *host = "http://172.20.10.9:4001/5";

- void setup() {

- Serial.begin(115200);

- delay(10);

- Serial.println();

- Serial.print("Connecting to ");

- Serial.println(wifiName);

- WiFi.begin(wifiName, wifiPass);

- while (WiFi.status() != WL_CONNECTED) {

- delay(500);

- Serial.print(".");

- }

- Serial.println("");

- Serial.println("WiFi connected");

- Serial.println("IP address: ");

- Serial.println(WiFi.localIP());   //You can get IP address assigned to ESP

- }

- void loop() {

- HTTPClient http;    //Declare object of class HTTPClient

- Serial.print("Request Link:");

- Serial.println(host);

- http.begin(host);     //Specify request destination

- int httpCode = http.GET();            //Send the request

- String payload = http.getString();    //Get the response payload from server

- Serial.print("Response Code:"); //200 is OK

- Serial.println(httpCode);   //Print HTTP return code

- Serial.print("Returned data from Server:");

- Serial.println(payload);    //Print request response payload

- if(httpCode == 200)

- {

- // Allocate JsonBuffer

- // Use arduinojson.org/assistant to compute the capacity.

- const size_t capacity = JSON_OBJECT_SIZE(10) + JSON_ARRAY_SIZE(8) + 60;

- DynamicJsonBuffer jsonBuffer(capacity);

- // Parse JSON object

- JsonObject& root = jsonBuffer.parseObject(payload);

- if (!root.success()) {

- Serial.println(F("Parsing failed!"));

- return;

- }

- // Decode JSON/Extract values

- Serial.println("===============Your Res as Info===============");

- Serial.println(F("Response:"));

- Serial.println("Name of Country:");

- Serial.println(root["name"].as<char*>());

- Serial.println(root["country"].as<char*>());

- Serial.println("Description:");

- Serial.println(root["description"].as<char*>());

- Serial.println("Latitude:");

- Serial.println(root["latitude"].as<char*>());

- Serial.println("Longitude:");

- Serial.println(root["longitude"].as<char*>());

- }

- else

- {

- Serial.println("Error in response");

- }

- http.end();  //Close connection

- delay(5000);  //GET Data at every 5 seconds

- }

مثال سوم) روشن و خاموش کردن چراغ توسط برد آردواینو توسط GPIO

- /*

- *  This sketch demonstrates how to set up a simple HTTP-like server.

- *  The server will set a GPIO pin depending on the request

- *    http://server_ip/gpio/0 will set the GPIO2 low,

- *    http://server_ip/gpio/1 will set the GPIO2 high

- *  server_ip is the IP address of the ESP8266 module, will be

- *  printed to Serial when the module is connected.

- */

- #include <ESP8266WiFi.h>

- const char *ssid = "sofa";  //ENTER YOUR WIFI SETTINGS

- const char *password = "AL20re/*";

- // Create an instance of the server

- // specify the port to listen on as an argument

- WiFiServer server(80);

- void setup() {

- Serial.begin(9600);

- delay(10);

- // prepare GPIO2

- pinMode(D4, OUTPUT);

- digitalWrite(2, 0);

- // Connect to WiFi network

- Serial.println();

- Serial.println();

- Serial.print("Connecting to ");

- Serial.println(ssid);

- //------------------------------------

- Serial.print("ESP Board MAC Address:  ");

- Serial.println(WiFi.macAddress());

- //------------------------------------

- WiFi.mode(WIFI_STA);

- WiFi.begin(ssid, password);

- while (WiFi.status() != WL_CONNECTED) {

- delay(500);

- Serial.print(".");

- }

- Serial.println("");

- Serial.println("WiFi connected");

- // Start the server

- server.begin();

- Serial.println("Server started");

- // Print the IP address

- Serial.println(WiFi.localIP());

- //  server.on("/", handleRoot);

- }

- void loop() {

- bool x=0;

- // Check if a client has connected

- WiFiClient client = server.available();

- if (!client) {

- return;

- }

- // Wait until the client sends some data

- Serial.println("new client");

- while(!client.available()){

- delay(1);

- }

- // Read the first line of the request

- String req = client.readStringUntil('\r');

- Serial.println("req=---------------------");

- Serial.println(req);

- client.flush();

- //http://192.168.1.100/gpio/1

- // Match the request

- if (req.indexOf("/gpio/0") != -1)

- {

- digitalWrite(2, HIGH);//off

- x=1;

- }

- else if (req.indexOf("/gpio/1") != -1)

- {

- digitalWrite(2, LOW);//on

- x=0;

- }

- else {

- Serial.println("invalid request");

- client.stop();

- return;

- }

- // Set GPIO2 according to the request

- client.flush();

- String convertableLocalIp = WiFi.localIP().toString().c_str();

- String url0 = "/gpio/0";

- String url1 = "/gpio/1";

- //    String toggleUrl = x== 1 ? '/gpio/1' : x == 0? '/gpio/0': '';

- Serial.println(x);

- // Prepare the response

- String s = "HTTP/1.1 200 OK\r\nContent-Type: text/html\r\n\r\n<!DOCTYPE HTML>\r\n<html> <button onclick='toggle()'>Toggle</button> <button onclick='on0()'>gpio0</button> <button onclick='on1()'>gpio1</button>  \r\nGPIO is now <script> function on0(){location.replace(\""+url0+"\")} function on1(){location.replace(\""+url1+"\")} function toggle(){location.replace(\""+x+"\"== 1 ? '/gpio/1' : \""+x+"\" == 0? '/gpio/0': '')} </script> ";

- s += (x)?"high":"low";

- s += "</html>\n";

- // Send the response to the client

- client.print(s);

- Serial.println(s);

- delay(1);

- Serial.println("Client disonnected");

- // The client will actually be disconnected

- // when the function returns and 'client' object is detroyed

- }